Lect 1

Materials used for aircraft and airplanes (structures)

Prof. Janusz Ćwiek,

Janusz.cwiek@polsl.pl, s. 2074

Platforma zdalnej edukacji

Requirements for aircraft materials and constructions:

- stiff;

- durable;

- strong;

- light.

Kinds of bonds:

- Primary (strong)

- covalent;

- metallic;

- ionic.

- Secondary (weak)

- hydrogen;

- van der waals.

| Bonds | Energy [kJ/mol] |
| --- | --- |
| ionic | 600-1550 |
| covalent | 500-1250 |
| metallic | 100-850 |
| Van der waals | <40 |

Melting temp is directly correlated with bond strength!

Classifications:

- Due to dominant bonds (all are strong bonds)

- metals, alloys (metallic bonding);

- ceramics (ionic and covalent bonding);

- polymers (covalent and van der waals bonding);

- composites.

- Due to origin

- natural materials;

- non-natural materials.

- Due to distribution of atoms/ions in the volume

- crystalline;

Metal properties:

- conductive electrically (G decreases with temperature);

- conductive thermally;

- ductile (tough);

- they possess a sheen;

- they lack transparency.

Ductility – the ability of a metal to deform without cracks appearing.

Toughness – the ability to absorb energy upon cracking.

Brittleness – the ability to crack upon any deformation.

Metallic bonds:

The outer shell releases electrons, and the electrons form a cloud inside a material.
Ion cores are positively charged and are attracted to the electron cloud, forming a bond.
Weaker than ionic and covalent bonds.

First metal used was copper, then bronze (alloy tin/copper). Then, iron was used. Then, aluminum. In the end, titanium.

Importance of metals:

- satisfies many design requirements;

- refined and known production processes;

- understood by engineers;

- stiff and strong;

- possibility of alloy creation;

- tough – able to absorb energy efficiently;

- steel – affordable in comparison to other engineering materials.

Important metals:

| Metal | Density [Mg/m^3] | Melting temp [degree C] |
| --- | --- | --- |
| Steel [Fe/C] | 7.87 | 1538 |
| Al | 2.7 | 660 |
| Cu | 8.96 | 1084 |
| Ti | 4.5 | 1668 |

Production of metals:

| Metal | Annual prod [Mtons] | Energy consum. [GJ/ton] |
| --- | --- | --- |
| Steel [Fe/C] | 1200 | 50 (ore) 17 (scrap) 35 (oxygen convector furn.) |
| Al | 63 20 | 300 (bauxite) 15 (recycling, scrap) |
| Cu | 21 | 90 (pyrometallurgy) |
| Ti | 0.22 | 900 (ore) |
| polymers | 300 | 100 |

Alloys:

- in engineering, pure metals aren’t used in general. only pure titanium is used;

- alloying increases strength, hardness, stiffness;

- to obtain corrosion-resistant steel, one must add chromium and nickel.

91 of 118 elements in the periodic table are metals, and 7 are metalloids.

Crystalline structure:

Unit cell – a region (parallel-sided) from which the crystal can form.
Smallest unit cell with the greatest symmetry is chosen.
Atoms pack according to the greatest efficiency (they lack directionality).
In close/closest packing, each atom of a metal has 12 neighbors. The number of nearest neighbors – coordination number.

All metals and most ceramics are crystalline.

Unit cells for metals:

- bcc (cube, corners and center)

- fcc (cube, corners and center of each face)

- hcp (hexagon, two layers + space between them, densely)

Ceramics have more shapes of structure.

Alotropy/polymorphism:

Alotropy – the ability of an element to exist in more than 1 crystal graphic form.

Polymorphism – broader

| Crystalline structure | Metals (and allotropic forms) |
| --- | --- |
| A1 | Au, Ag, Pt, Pb, Cu, Al, Ni, Fe (gamma), Co (alpha) |
| A2 | Cr, Mo, Nb, W, V, Ti (beta), Fe (alpha), Zr (beta) |
| A3 | Mg, Zn, Cd, Ti (alpha), Co (epsilon), Zr (alpha) |

Low-temp allotropic forms: Fe (alpha), Ti (alpha), Co (alpha), Zr (alpha)

High-temp allotropic forms: Fe (gamma), Ti (beta), Co (epsilon), Zr (beta)

A1 – fcc, A2 – bcc, A3 – hcp.

bcc (A2) Fe (alpha)

Atoms realign upon a change to the metal’s temperature. If the temperature of change remains constant, an allotropic transformation occurs. Iron:

1394

T

fcc (A1) Fe (gamma)

912

bcc (A2) Fe (alpha)

Titanium

liquid

1668

Ti (beta, bcc)

882

Ti (alpha, hcp)

Alpha – high-temp structure

Greek letter not correlated to shape of the crystalline form and allotropy.